# Intermodal Yard Simulation – Initial Data & Assumptions

This document defines the initial real-world assumptions and data anchors for the Intermodal Container Operations Decision Support System. These assumptions are based on public freight statistics and are intended to model a realistic mid-size U.S. intermodal rail terminal.

## 1. Terminal Profile

The simulation models a mid-size U.S. intermodal rail terminal.

- Containers handled per day: approximately 2,000
- Operating window: 24 hours
- Average arrival rate: 80–90 containers per hour
- Peak arrival rate: up to 140 containers per hour during surge periods

This scale reflects real intermodal terminals without modeling mega-port complexity.

## 2. Simulation Time Configuration

- Total simulation duration: 24 hours
- Time step granularity: 5 minutes

This provides sufficient resolution to observe congestion and retrieval behavior.

## 3. Container Arrival Model

Containers arrive continuously throughout the day.

- Base arrival rate: approximately 1.4 containers per minute
- Peak arrival rate: approximately 2.3 containers per minute
- Two daily surge windows (morning and evening)

Arrivals are synthetic but calibrated to public freight throughput patterns.

## 4. Yard Layout and Capacity

The yard is modeled as a finite set of container stacks.

- Number of stacks: 120
- Maximum stack height: 5 containers
- Total yard capacity: 600 containers

This configuration intentionally creates congestion pressure and forces trade-offs.

## 5. Outbound Train Schedule

Outbound trains create departure deadlines for containers.

- Number of outbound trains per day: 4
- Train departure times: 06:00, 12:00, 18:00, 23:00
- Capacity per train: 350 containers

Total outbound capacity is intentionally lower than inbound volume to create dwell accumulation.

## 6. Container Urgency Definition

Each container is assigned a target outbound train upon arrival.

- A container becomes urgent two hours before its assigned train departs
- Containers not retrieved and loaded by departure time are considered missed connections

Urgency directly drives retrieval priority and rehandle risk.

## 7. Initial Key Performance Indicator

The initial KPI for the simulation is:

Rehandles per Container

Definition: The average number of additional container moves required because a target container was blocked by other containers at retrieval time.

This KPI directly measures yard inefficiency caused by poor placement decisions.

## 8. Baseline Decision Strategy

The baseline strategy provides a comparison point for future improvements.

Placement Rule:
- When a container arrives, place it on the currently shortest available stack.

This strategy does not consider urgency, departure times, or future retrieval needs.